APPROVED

By Decision of the Customs Union

Commission No. 799

of September 23, 2011

TECHNICAL REGULATION OF THE CUSTOMS UNION

TR TS 009/2011

On Safety of Perfumes and Cosmetics

Contents

| Section 1. Scope of Application………………………………………………………………….. | 2 |
| --- | --- |
| Section 2. The Rules for Identification of Perfumes and Cosmetics…………………………….. | 2 |
| Section 3.Terms and Definitions…………………………………………………………………. | 2 |
| Section 4. The Rules of Market Circulation………………….…………………………………... | 4 |
| Section 5. Requirements to Perfumes and Cosmetics ……………………………………………. | 4 |
| Section 6. Conformity Assessment………….……………………………………………………. | 9 |
| Section 7. Marking with a Single Mark for Products Circulation on the Market of CU Member States …………………………………………………………………………………… | 11 |
| Section 8. Disclaimer…………………………………………………………………………….. | 12 |
| Section 9. Government Supervision …………………………………………………..…………. | 12 |

II

Section 1. Scope of Application

1. This Technical Regulation of the Customs Union (hereinafter referred to as CU) shall apply to the consumer packaged perfumes and cosmetics released for circulation on the territory of CU member-states.

2. This Technical Regulation of CU shall not apply to the products meant for swallowing, inhalation, injection or implantation in the human body, permanent cosmetics applied with disturbance of the skin cover, as well as to the products used for diagnostics and treatment of diseases.

3. This Technical Regulation of CU specifies requirements to the products and to the manufacturing processes related thereto for protection of the human life and safety, property, environment protection, as well as for prevention of any activities misleading the consumers regarding purpose and safety of the products.

Section 2. The Rules for Identification of Perfumes and Cosmetics

1. For the purpose of this Technical Regulation of CU, identification shall be carried out according to the characteristics peculiar to perfumes and cosmetics, and specified in this Technical Regulation of CU.

2. Characteristics of perfumes and cosmetics shall be as follows:

- the method of products application – perfumes and cosmetics are meant exclusively for external application on certain parts of the human body;

- the place of products application – perfumes and cosmetics are applied on skin, hair, nails, lips, teeth, mucous membrane of the mouth cavity and external genitals;

- the goals of products application (either individually or in any combination) – cleansing and/or modification of the appearance of skin, hair, nails, lips, teeth, mucous membrane of the mouth cavity and external genitals without violation of integrity thereof; odorizing thereof and/or smell correction; maintenance thereof in good functional status.

3. Identification of perfumes and cosmetics for the purpose of this Technical Regulation of CU shall be carried out according to the documentation. Such documentation may include any technical documents, and/or supply contracts, and/or specifications, and/or labels, and/or abstracts, and other documents characterizing the products.

Section 3. Terms and Definitions

The following terms used in this Technical Regulation of CU shall have the following meanings:

ampule cosmetics – perfumes and cosmetics packaged in a hermetically sealed glass (polymer) container, and meant for single application;

safety of perfumes and cosmetics – the whole range of features and characteristics of perfumes and cosmetics which ensure the absence of any adverse effect on consumers exerted by perfumes and cosmetics when used in accordance with the purpose and application method during the shelf-life thereof;

secondary packaging – the packaging in which primary packaged perfumes and cosmetics are put; not a transport packaging;

release for circulation – the moment (including the first offer), starting from which the manufacturer or the seller (importer) destines the products fore sales or another transfer to consumers on the territory of CU member-states;

manufacturer – a legal entity or an individual entrepreneur manufacturing perfumes and cosmetics and/or releasing perfumes and cosmetics for circulation under its own name and/or trademark, and responsible for compliance thereof with requirements of this Technical Regulation of CU;

importer – a resident of CU member-state, which has signed a foreign trade agreement on the transfer of perfumes and cosmetics with a non-resident of CU member-state, sells these perfumes and cosmetics, and is responsible for compliance thereof with requirements of this Technical Regulation of CU;

ingredient of perfumes and cosmetics (ingredient) – a substance or a mixture of substances of synthetic or natural origin used for the manufacture of perfumes and cosmetics. Ingredients do not include any impurities in ingredients or any materials used in the manufacture of perfumes and cosmetics, but missing in the finished product;

preservatives – chemical substances of natural and/or synthetic origin providing for the resistance of perfumes and cosmetics to microbial contamination during the shelf-life thereof;

decorative cosmetics – perfumes and cosmetics for the makeup of face, eyes, lips, eyebrows, body, as well as for dyeing of hair and nails;

baby cosmetics – perfumes and cosmetics for children under the age of 14 years;

intimate cosmetics – perfumes and cosmetics for the care of external genitals and surrounding body areas;

professional cosmetics – perfumes and cosmetics to be used by legal entities and individual entrepreneurs when rendering the hairdressing and/or beauty services;

permanent cosmetics – perfumes and cosmetics for the application of patterns on skin without injections;

dyes – chemical substances of natural or synthetic origin meant for the coloring of perfumes and cosmetic, or forming part of the perfumes and cosmetics for dyeing of skin, hair and nails;

marking of perfumes and cosmetics (marking) – information for consumers in the form of inscriptions, digital, color and graphic symbols to be applied on consumer packaging, label, ticket;

name of perfumes and cosmetics – verbal and/or digital product designation assigned by the manufacturer;

purpose of perfumes and cosmetics – functional property of perfumes and cosmetics specifying the scope of application thereof;

type of perfumes and cosmetics – designation of the uniform perfumes and cosmetics (toothpaste, lotion, perfume, cream etc.);

nanomaterial – an insoluble or biologically stable and specially produced material having at least one external dimension or internal structure of 1 to 100 nm;

uniform perfumes and cosmetics – the products of one type, similar in composition and meeting the same requirements;

perfumes and cosmetics (PAC) – a substance or mixtures of substances meant for application directly on outer covering of the human body (skin, hair, nails, lips and external genitals) or on teeth and mucous membrane of the mouth cavity for the single or key purpose of cleansing or modification of appearance thereof, odorizing, and/or correction of the body odors, and/or protection thereof, and/or maintenance thereof in good condition, and/or care thereof;

primary packaging – the packaging that contacts perfumes and cosmetics directly;

peeling – perfumes and cosmetics for removal of the surface epidermis cells with the use of keratolytics;

consumer packaging of perfumes and cosmetics – primary or the combination of primary and secondary packaging of perfumes and cosmetics which reaches consumers together with perfumes and cosmetics, without serving as the shipping container;

tester – a sample of perfumes and cosmetics presented in a miniature packaging and/or in a simplified packaging, and meant for testing and approval;

formula – the full list of ingredients that form part of perfumes and cosmetics specified by the manufacturer, indicating the mass fraction of ingredients;

sachet – perfumes and cosmetics in disposable flexible consumer containers;

oral hygiene products – hygiene and/or preventive care perfumes and cosmetics for direct application on teeth, gums and mucous membrane of the oral cavity with the single and/or key purpose of cleansing, odorizing, modification of appearance, protection, maintenance thereof in good condition;

shelf-life – the period upon the expiry of which perfumes and cosmetics are deemed unsuitable for intended use thereof. Shelf-life is specified by the products manufacturer in technical documents. During the products shelf-life, the manufacturer must ensure products compliance with the requirements to safety of consumers’ life and health, and preservation of consumer properties subject to the storage conditions;

standard conditions for the storage of perfumes and cosmetics – the temperature for storage of liquid products: from 5°C to 25°C; of hard toilet soap: 5°C below zero or higher; of other perfumes and cosmetics: from 0°C to 25°C; the absence of direct sunlight;

technical documents – the documents according to which perfumes and cosmetics are manufactured, stored, transported (technical conditions, standards, manufacturing instructions, formulas, specifications etc.);

shipping containers – containers meant for packaging, storage and transportation of consumer packaged perfumes and cosmetics, and forming an independent transport unit;

authorized representative of the manufacturer – a legal entity or an individual entrepreneur duly registered in CU member-state and authorized by the manufacturer to act on behalf of the latter upon conformity assessment and release of perfumes and cosmetics for market circulation, and responsible for compliance thereof with requirements of this Technical Regulation;

UV filter (ultraviolet filter) – a substance meant to protect skin against the UV irradiation by means of absorption, reflection or diffusion;

label – a means of information about the packed perfumes and cosmetics, located on the consumer packaging;

ticket – an item of arbitrary shape, size and material meant for marking, attached to or applied on one unit of perfumes and cosmetics or put in the packaging.

Section 4. The Rules of Market Circulation

1. Perfumes and cosmetics shall be released for market circulation subject to compliance thereof with this Technical Regulation, and other technical regulations of CU applicable thereto, no additional requirements thereto being specified besides those contained in this Technical Regulation of CU, and no additional procedures for conformity assessment (verification) being carried out.

2. Perfumes and cosmetics that fail to meet requirements of this Technical Regulation shall not be marked with the single mark for products circulation on the market of CU member-states, and shall not be placed on the market.

Section 5. Requirements to Perfumes and Cosmetics

1. The safety of perfumes and cosmetics shall be ensured by a set of requirements:

1) to the composition;

2) to the physicochemical characteristics;

3) to the microbiological characteristics;

4) to the content of toxic elements;

5) to the toxicological characteristics;

6) to the clinical (clinical and laboratory) characteristics;

7) to manufacture;

8) to the consumer packaging;

9) to the products marking.

2. Requirements to the composition of perfumes and cosmetics

2.1. the use of substances in perfumes and cosmetics shall be prohibited according to Annex 1;

2.2. the use of substances in perfumes and cosmetics shall be allowed subject to the specified restrictions according to Annex 2;

2.3. the use of dyes in perfumes and cosmetics shall be allowed according to Annex 3;

2.4. the use of preservatives in perfumes and cosmetics shall be allowed according to Annex 4;

2.5. the use of UV filters in perfumes and cosmetics shall be allowed according to Annex 5.

2.6. The lists of ingredients of perfumes and cosmetics set forth in Annexes 1 – 5 shall be updated in the specified manner.

3. Requirements to the physicochemical characteristics of perfumes and cosmetics

The value of hydrogen ion concentration (pH) in the perfumes and cosmetics listed in Annex 6 must comply with the standards specified in the said Annex.

The content of fluorine (expressed as the molar weight of fluorine) in the fluorine-containing oral hygiene products must meet the requirements specified in Annex 2.

4. Requirements to the microbiological characteristics of perfumes and cosmetics

4.1. Microbiological characteristics of perfumes and cosmetics must meet the requirements specified in Annex 7.

4.2. Microbiological characteristics shall not be determined for the following types of perfumes and cosmetics:

1) perfumes and cosmetics containing ethyl alcohol and/or any organic solvents in the concentration of more than 25% of volume, and used without dilution;

2) nail lacquers with the exception of water-based nail lacquers;

3) deodorants, deodorants – antiperspirants, antiperspirants;

4) oxidative hair dyes, lightening and highlighting products;

5) hair perm and straightening products based on thiol compounds;

6) depilation products based on thioglycolic acid;

7) fat-based hard toilet soap;

8) dry liners;

9) bath salts;

10) 100% essential oils;

11) teeth whitening products containing hydrogen peroxide or other components releasing hydrogen peroxide, including carbamide peroxide and zinc peroxide, the concentration of hydrogen peroxide (as ingredient or released) amounting to 0.1% - 6.0%;

12) shaving products (creams, gels etc.) having hydrogen ion concentration (pH) of more than 10.0.

5. Requirements to the content of toxic elements

The content of toxic elements in the perfumes and cosmetics containing more than 1% of raw materials of the natural plant origin or natural mineral origin, must not exceed: arsenic – 5.0 mg/kg; mercury – 1.0 mg/kg; lead – 5.0 mg/kg.

6. Requirements to the toxicological and clinical (clinical and laboratory) characteristics of perfumes and cosmetics

6.1. Perfumes and cosmetics must be safe for humans when used according to the intended purpose thereof.

6.1.1. Toxicological characteristics of perfumes and cosmetics must meet the requirements specified in Annex 8.

6.1.2. Clinical (clinical and laboratory) characteristics of perfumes and cosmetics must meet the requirements specified in Annex 9.

6.1.3. Clinical characteristics of the oral hygiene products must meet the requirements specified in Annex 10.

6.1.4. Toxicological testing of decorative cosmetics, including nail lacquers, as well as hair dyeing and toning products of the same type and name, manufactured according to a single technical document, but differing in tone, shall be carried out in full on a sample of at least 30% of tones claimed for study. Test specimens must contain the maximum amount of each specific dye. For other tones claimed for study, only the sensitizing effect shall be determined.

6.1.5. Toxicological and clinical (clinical and laboratory) characteristics of perfumes and cosmetics shall be determined in the accredited test laboratories (centers) following the positive results of physicochemical and microbiological testing of the products.

7. Requirements to the manufacture of perfumes and cosmetics

7.1. Requirements to the manufacturing processes

To ensure the safety of perfumes and cosmetics during the manufacturing process, it is necessary:

- to carry out manufacture thereof in line with the requirements of Clause 7.2 of this Technical Regulation;

- to carry out manufacture thereof in production facilities meeting the requirements of Clause 7.3 of this Technical Regulation;

- to use manufacturing equipment and tools meeting the requirements of Clause 7.4 of this Technical Regulation;

- to involve in the products manufacture the staff that meets the requirements of Clause 7.5 of this Technical Regulation;

7.2. Ensuring safety of perfumes and cosmetics in the process of manufacture thereof

The safety of products in the process of manufacture and circulation thereof shall be ensured by:

- selecting the products manufacturing processes which are necessary to ensure the products safety;

- selecting consistency and flow of the products manufacturing processes which eliminates pollution of raw materials, packaging materials, bulk and finished products;

- determining the controlled stages of manufacturing processes and products at the stages of manufacture thereof within the manufacture control programs;

- exercising control over the raw materials, manufacturing equipment and auxiliary materials used in the products manufacture, as well as control over bulk and finished products by the means ensuring the required accuracy and completeness of control;

- specifying requirements to the raw materials, packaging materials, acceptance criteria, actions in the event of defect or minor deviations, transportation conditions;

- exercising control over the operation of manufacturing equipment so as to ensure manufacture of the products that meet the requirements of this Technical Regulation;

- documenting the information on controlled stages of manufacturing processes and the results of control over perfumes and cosmetics at the stages of manufacture thereof;

- recording and registering in detail of all manufacturing operations for each stage (specifying the added raw materials, temperatures, speeds, period of mixing, sampling, cleaning, equipment disinfection, if needed, and transfer of bulk products)

- observing the terms of products storage;

- maintaining the production facilities, manufacturing equipment and tools used in the products manufacturing process in a condition that would prevent the products pollution;

- selecting the modes of personal hygiene observance by employees with a view of ensuring the products safety;

- using the modes and frequency of cleaning, washing, disinfection and fumigation of the production facilities, manufacturing equipment and tools used in the manufacture of perfumes and cosmetics, which ensure the safety of bulk and finished products;

- maintaining and keeping the documents in paper and (or) electronic format confirming compliance of the manufactured products with requirements of this Technical Regulation;

- tracing perfumes and cosmetics;

- exercising control over the finished products according to the specified methods of testing for compliance with the acceptance criteria, before placement thereof on the market.

- keeping archival samples of each batch of perfumes and cosmetics.

7.3. Requirements to the production facilities where perfumes and cosmetics are manufactured

7.3.1. Production facilities must meet the following requirements:

- ventilation and/or air conditioning systems installed in production facilities must allow clean air into the production room; the air should not be a source of products pollution;

- lighting systems installed in production facilities must allow for sighting any fragments and preventing them from getting into products; they also should not be a source of products pollution;

- production facilities should not have rooms for storage of the personal and working (special) clothing of staff;

- production facilities should not have toilets; toilet doors should not open directly into production facilities;

- layout of production facilities should allow for consistency and flow of the production processes, excluding counter-flows or cross-flows of raw materials, bulk and finished products, unclean and clean equipment, if necessary to prevent products pollution;

- production facilities must be arranged so as to prevent penetration of animals, including rodents and insects;

- production facilities must be arranged so as to exclude zones where dirt, condensation, mold and other similar sources of products pollution cannot be removed;

- construction of floors should prevent the accumulation of water in production facilities;

- surfaces of floors, walls and doors should be made of waterproof, washable and non-toxic materials that can be sanitized and disinfected;

- ceilings (or inner surface of the roof in the absence of ceilings) and constructions located above the production facilities should not allow any particles or condensate to get into products;

- opening windows (skylights) must be fitted with insect screens;

- sewage equipment in the production facilities must be designed and constructed so as to eliminate the risk of products pollution.

7.3.2. It is not allowed to store any substances and materials not used in the manufacture of perfumes and cosmetics, with the exception of detergents and disinfectants, in production facilities. Detergents and disinfectants may be stored in the designated areas of production facilities.

7.4. Requirements to the manufacturing equipment and tools used in the manufacture of perfumes and cosmetics

7.4.1. Manufacturing equipment and tools used in the manufacture of perfumes and cosmetics must:

- allow for their sanitization and disinfection;

- be made of materials that are not a source of products pollution.

7.4.2. Manufacturing equipment, if necessary to achieve the goals of this Technical Regulation, shall be provided with the corresponding monitoring devices.

7.5. Requirements to staff involved in the manufacture of perfumes and cosmetics

7.5.1. Staff involved in the manufacture of perfumes and cosmetics though direct contact with raw materials, bulk and finished products, materials and supplies should observe personal hygiene, wear clean special production clothes and shoes to the extent it is necessary in order not to be a source of products pollution.

7.5.2. Staff involved in the manufacture of perfumes and cosmetics cannot bring items that are not used in performance of their duties and can be a source of products pollution to production facilities, smoke and eat in production facilities.

8. Requirements to consumer packaging of perfumes and cosmetics

8.1. Consumer packaging must ensure the safety and integrity of perfumes and cosmetics throughout the shelf-life of products.

9. Requirements to marking of perfumes and cosmetics

9.1. Marking of perfumes and cosmetics shall be done by applying consumer information in the form of inscriptions, digital, color and graphic symbols on consumer packaging, labels or tickets in line with the marking requirements specified in Annexes 2, 3, 4, 5. If a perfume or cosmetic product is provided with accompanying information (ticket), a graphic sign shall be applied on the product in the form of a hand on an open book (Annex 11).

9.2. Marking of perfumes and cosmetics should contain the following information:

- type and name (if any) of perfumes and cosmetics;

- purpose of perfumes and cosmetics, if not implied by the type of products;

- baby cosmetics should have the appropriate information in the marking;

- manufacturer’s name and location (registered address, including country);

- country of origin of perfumes and cosmetics (unless the country where production is located coincides with the registered address of the manufacturer);

- name and location (registered address) of the company authorized by the manufacturer to receive customer claims (manufacturer’s authorized representative or importer), unless the manufacturer receives claims personally on the territory of CU member-state;

- nominal quantity (volume or weight) of products in consumer packaging (for hard toilet soap – the nominal mass of a piece at the time of packaging), except for perfumes and cosmetics with a nominal mass of less than 5 g or nominal volume of less than 5 ml, or perfume and cosmetic testers;

- color and/or tone (for decorative cosmetics and dyes);

- mass fraction of fluoride (% or mg/kg or ppm) for oral hygiene products containing fluoride compounds;

- shelf-life:

date of manufacture (month and year) and shelf-life (months, years), or the inscription “best before” (month, year) or “use before” (month, year);

- description of the storage conditions if different from the standard;

- special precautions (if any) in the use of products, including warnings specified in Annexes 5.2 of this Technical Regulation;

- batch number or special code to identify the batch of perfumes and cosmetics;

- information on the modes of use of perfumes and cosmetics, the lack of which might lead to the consumer’s misuse of the products;

- list of ingredients in accordance with Clause 9.3 of this Technical Regulation.

9.3. The list of ingredients must be preceded by the heading “Ingredients” or “Composition”.

Ingredients shall be listed in decreasing order of their mass fraction in the formula, and the perfume (aroma) blend shall be specified as a single ingredient without revealing its composition. If the product contains any ingredients (No. 67-92) specified in Annex 2 and their content is more than 0.01% for rinse-off products and 0.001% for leave-in products, they should be listed in the composition.

Ingredients present in the form of nanomaterials shall be clearly specified in the list of ingredients, with the word «nano» following the product name in brackets, if ingredients are listed in accordance with the International Nomenclature of Cosmetic Ingredients (INCI).

Ingredients in concentrations of less than 1% may be listed in any order after the ingredients in concentrations of more than 1%.

Dyes can be listed in any order after the other ingredients according to the color index or conventions used.

The list of ingredients can be presented either in the national/official language(s) of CU member-states where perfumes and cosmetics are sold or in accordance with the International Nomenclature of Cosmetic Ingredients (INCI) using the Latin alphabet.

Packaging of decorative cosmetics issued as a series of different tones may enumerate all dyes used in the series using the term “may contain” or the sign (+/-).

9.4. Information referred to in Clause 9.2 of this Section shall be readable and remain on the packaging provided that the products are used as intended.

9.5. Information about perfumes and cosmetics referred to in Clause 9.2 of this Section shall be presented in the national language(s) of CU member-states where the perfumes and cosmetics are sold.

Manufacturer’s name and location, and product name may be written using the Latin alphabet. The country of origin of perfumes and cosmetics shall be specified in the national/official language(s) of CU member-states where the perfumes and cosmetics are sold.

Section 6. Conformity Assessment

1. Conformity of perfumes and cosmetics with the requirements of this Technical Regulation of CU shall be assessed by the manufacturer (manufacturer’s authorized representative, importer) duly registered in CU member states.

2. Conformity assessment of perfumes and cosmetics, with the exception of perfumes and cosmetics listed in Annex 12, shall be carried out by making a declaration of conformity based on the evidence obtained jointly with a testing laboratory (center) duly accredited in CU member states and on the own evidence.

The declaration of conformity of perfumes and cosmetics with this Technical Regulation and the certificate of state registration of products shall be issued for one or more names of products of the same type, and shall be valid until changes are made to the name and/or formula of the products, leading to a change in safety characteristics

3. Conformity assessment of perfumes and cosmetics listed in Annex 12 shall be carried out by means of state registration of the products, confirmed by issuance of the relevant registration certificate.

4. State registration of the perfumes and cosmetics manufactured in the Customs Union shall be carried out at the stage of their launch into manufacture, while state registration of the perfumes and cosmetics imported into the customs territory of the Customs Union – before entry thereof into the customs territory of the Customs Union.

State registration of products shall be carried out by the public sanitation and disease control authority (hereinafter referred to as the registration authority) authorized by the relevant party.

To receive the certificate of state registration, the applicant (manufacturer, manufacturer’s authorized representative, importer) shall submit the following documents:

- application;

- copies of documents according to which the products are manufactured (technical documents and/or the list of ingredients specifying concentrations of the ingredients listed in Annexes 2-5), certified by the applicant;

- manufacturer’s written notice that their products comply with the documents according to which they are manufactured (manufacturer’s certificate of quality attested by manufacturer or an official letter of the manufacturer can serve as such notification);

- copies of documents stating organoleptic and physicochemical characteristics of products, certified by the applicant.

In case the composition of perfumes and cosmetics contains any nanomaterials, the manufacturer shall provide information on such nanomaterial, including its chemical name, particle size and physicochemical properties;

- a sample of consumer packaging marking for perfumes and cosmetics, confirming compliance with the requirements of Clause 9 Section 5 of this Technical Regulation;

- annotation containing the stated consumer properties (if stated by manufacturer in the products marking), special precautions (if any) during the use of products, and information on uses of perfumes and cosmetics, the lack of which could lead to consumer’s misuse of the products;

- research (testing) reports or health research reports, or scientific reports, or expert opinions on compliance with the requirements of Clauses 3 – 6 Section 5 of this Technical Regulation, issued by an accredited testing laboratory (center);

- the manufacturer’s document on the manufacture process conformity with the requirements of Clause 7 Section 5 of this Technical Regulation of CU (manufacturer’s written notice on the manufacture process conformity with this Technical Regulation of CU, or manufacturer’s declaration (statement or written notice) of compliance with the principles of GMP, or a certificate of conformity for the quality management system, or a certificate of conformity of perfumes and cosmetics manufacture with the principles of good manufacturing practice (GMP));

- documentary evidence of the consumer properties of perfumes and cosmetics specified in consumer packaging marking (antimicrobial, anti-wrinkle effect, SPF-factor, anticariogenic activity, anti-inflammatory effect of oral hygiene products etc.), certified by the applicant.

Consumer properties of perfumes and cosmetics may be confirmed by means of volunteer testing, and/or research of samples, and/or instrumental methods, and/or on the basis of scientific data for the ingredients of perfumes and cosmetics;

- extract from the Unified State Register of Legal Entities or the Unified State Register of Individual Entrepreneurs.

Translations of the manufacturer’s documents from a foreign language into the national/official language of CU member state must be notarized or signed by the translator and accompanied by a copy of the certificate attesting the translator’s qualification.

The applicant shall be liable for the accuracy of documents submitted for obtaining a certificate of products safety.

Information on perfumes and cosmetics that have undergone state registration shall be recorded in the Unified Register of Certificates of State Registration.

State registration may be denied in cases specified in the CU law.

State registration may be cancelled by the registration authority in cases specified in the CU law.

5. The following documents must be submitted by the applicant (manufacturer, manufacturer’s authorized representative, importer) for each product name in justifying its declaration of conformity of perfumes and cosmetics with the requirements of this Technical Regulation of CU:

- list of ingredients of perfumes and cosmetics specifying concentrations of the ingredients listed in Annexes 2-5;

- copies of documents stating organoleptic and physicochemical characteristics of products, certified by the applicant;

- research (testing) reports or health research reports, or scientific reports, or expert opinions on compliance with the requirements of Clauses 3 –6 Section 5 of this Technical Regulation of CU, issued by an accredited testing laboratory (center);

- a sample of consumer packaging marking for perfumes and cosmetics, confirming compliance with the requirements of Clause 9 Section 5of this Technical Regulation of CU;

- manufacturer’s document on the manufacture process conformity to the requirements of Clause 7 Section 5 of this Technical Regulation (manufacturer’s written notice on the manufacture process conformity with this Technical Regulation of CU, or manufacturer’s declaration (statement or written notice) of compliance with the principles of GMP, or a certificate of conformity for the quality management system, or a certificate of conformity of perfumes and cosmetics manufacture with the principles of good manufacturing practice (GMP));

- documentary evidence of the consumer properties of perfumes and cosmetics specified in the consumer packaging marking (antimicrobial, anti-wrinkle effect, SPF-factor, anticariogenic activity, anti-inflammatory effect of oral hygiene products etc.), certified by the applicant.

Consumer properties of perfumes and cosmetics may be confirmed by means of volunteer testing, and/or research of samples, and/or instrumental methods, and/or on the basis of scientific data for the ingredients of perfumes and cosmetics.

Translations of manufacturer’s documents from a foreign language into the national/official language of CU member state must be notarized or signed by the translator and accompanied by a copy of the certificate attesting the translator’s qualification.

6. The declaration of conformity of perfumes and cosmetics with this Technical Regulation of CU shall be submitted to the accredited certification (conformity assessment/confirmation) authority for due registration.

Section 7. Marking with a Single Mark for Products Circulation on the Market of CU Member States

1. Perfumes and cosmetics meeting the requirements of this Technical Regulation of CU, which have undergone assessment of conformity with this Technical Regulation under Section 6 must be marked with the single mark for products circulation on the market of CU member states.

2. Products shall be marked with a single mark for products circulation on the market of CU member states prior to their release into the market.

3. The single mark for products circulation on the market of CU member states shall be applied on each product unit (consumer packaging, label, ticket) and/or shipping documentation.

The single mark for products circulation on the market of CU member states shall be applied in any manner providing a sharp and clear image during the entire shelf life of perfumes and cosmetics.

Section 8. Disclaimer

1. CU member states shall make all efforts to limit and/or prohibit the release for circulation of perfumes and cosmetics that endanger human life and health, and withdraw them from the customs territory of the CU member state.

2. The competent authority of a CU member state shall notify the CU Commission and competent authorities of other CU member states of its decision, specifying the reasons for such decision and providing information justifying the need for such measure.

3. Should the competent authorities of other CU member states object to the decision referred to in Clause 1 of this Section, the CU Commission shall immediately consult competent authorities of all CU member states to find a mutually acceptable solution.

Section 9. Government Supervision

Government supervision over compliance with this Technical Regulation on the territory of a CU member state shall be conducted according to the law applicable in such CU member state.